# Dock Winterization Procedures

# Dock A is the dock closest to the Boat Ramp.

# Dock B is the other dock.

# Under each dock A (near the start) is a drain valve and a shutoff valve for that dock.

# Under each dock B (near the start) is a shutoff valve for that dock. The drain is above the dock.

# You will need 16-18 gallons of anti-freeze.

- Preparing the docks for winterization.

  - Turn off Well Pump circuit breaker in the Pool House.

  - Close the valve outside the Pool House (near the meter leading into the Pool House).

  - Open the drain valves  (under the dock near the start) of each of the docks to drain the tank

  - Disconnect all hoses on the docks.

  - When the tank is drained, Close the drain valve and the shutoff valve under dock A.

- Clearing Dock B.

  - All the valves should be closed except one at the end of dock B.

  - At the start of dock B, connect the air compressor tank to the “drain” valve on dock B.

  - Blow thru the lines three times (or till sufficient).

  - Close the shutoff valve under dock B.

- Clearing the pressure tank and line.

  - Open the valve on the pool side of the pressure tank. (You may have to disconnect the hose.)

  - Keep the valve to the  Pool House closed

  - Blow thru the lines three times (or till sufficient).

  - Close the valve on the pool side of the pressure tank.

  - Close the shutoff valve at the tank leading down to the docks also.

- Clearing Dock A

  - Open the shutoff valve under dock A.

  - Open a valve at the end of dock A.

  - Blow thru the lines three times (or till sufficient).

- Winterizing with Anti-freeeze.

  - Disconnect the air compressor tank.

  - Connect a hose to the start of dock B and run it up to the top of the steps.

  - With a funnel pour the anti-freeze thru the hose at the top of the steps. (make sure there is a straight run i.e. no air locks)

  - Alternative to b,c (known as the Harold method) – Get a 5 gallon bucket and place Harold's sump pump in it. Connect a hose from the sump pump to the spicket near the start of B dock. Fill the bucket with anti-freeeze and start the pump.

  - Open the drain valve under dock A until you see anti-freeze coming thru then close it  and open the next valve until you see anti-freeze coming thru. Continue opening and closing the valves in sequence until you get to the end.

  - Open the shutoff valve under dock B and repeat the procedure for dock B. Opening until you see anti-freeze then closing it and opening the next valve in sequence until you get to the end.

# Dock Spring Preparation Procedures

- Make sure the valve outside the Pool House (near the meter leading into the Pool House) is closed.

- Turn on the Well Pump circuit breaker in the Pool House.

- Open the shutoff valve at the tank leading down to the docks.

- Open the shutoff valves under each dock.

- Check for leaks.

- If there are leaks fix them.